Vizsgaremek Angol szövegrész
Készítette: Dittera Károly Lajos
Osztály: 13./B
Szakmai csoport: IRAÜ1
Nyelvi csoport: AN_2

As we said previously, the servers attached to Project E.N.I.S. have been put into IBM’s central territory, attached to the backbone of IBM to be exact, within area 1 of the OSPF topology. Project E.N.I.S. is being developed simultaneously on Linux and Windows platforms, so that upon its release it’ll be compatible with both platforms. We established and configured multiple servers, separated into Linux and Windows, with each server providing a different function. There are Active Directories, File servers and DNS servers, just to name a few. Of course, for the sake of time, and for conserving our resources for this presentation smartly, we didn’t bring this full-scale topology here, we have simulated segments in packet tracer, and clearer full-value functions in our Virtual Machines.

The specific server I’ll talk about is our DHCP server, and before I get into how we made it work and with what configuration, I’ll explain the purpose of it. Dynamic Host Configuration Protocol, or DHCP for short, is a networking protocol that’s attached to the OSI models seventh layer, the application layer. This protocol uses the UDP data transfer protocol during its operation. A DHCP server can provide networking data to its clients, the data is based on a pool, which often has different syntax depending on what type of OS the server is using. However, the data that’s meant to be delivered stays the same. This data consists of networking prefixes and host ID-s, the subnets default-gateway, the address of the DNS server, both host names and domain names, and lastly leasing times. For enhanced security it can also provide the address of time servers.

The inner workings of a DHCP server differs a bit depending on the type of IP protocol used, v4 or v6. I’ll explain v4 first, here a 4-step process, which is referred to as DORA. Discover, Offer, Request and Acknowledge. These are the names of the different steps a DHCP client and server take to establish their connection and register the leasing of the network data provided. The first step, DHCP Discovery, is sent out by the client, it’s a broadcast message looking for a DHCP server that’ll provide them with information. The server that’s able to respond first, effectively a server that has a pool that corresponds with the subnets default-gateway, will send back a DHCP Offer message, providing them with a proposal of all the aforementioned network data. The client will send a reply message, the DHCP Request message, and inform the server that the proposed data isn’t faulty, and the client wants to lease it. Once the server has also made sure that the data isn’t faulty, it sends back the final DHCP Acknowledge message, with which it registers and confirms the leasing, for the configured duration, and provides the client with the final networking configuration details. There are some extra DHCP messages that are kept for more special cases. The DHCP NAK message is sent by the server to the client if the client’s request cannot be followed trough, for example if pool/scope has been misconfigured or deactivated. DHCP Decline is sent by the client towards the server to indicate that the IP address that was proposed is already in use, typically because of a duplicate address conflict. The last most used DHCP special mission is DHCP release, is sent out by the client, and lets the server know that they are giving up on the leftover lease duration. Once the server receives this, they unregister the information, and it becomes available to be dispatched again.